# ASSESSING THE IMPACT OF HISTORICAL AND SYNTHETIC DATA ON DEEP LEARNING-BASED VOLTAGE STABILITY MONITORING IN SCARCE DATA ENVIRONMENTS: A COMPARATIVE REVIEW

By

YABANI GELWASA GALADIM1 and PEACE IFEOLUWA IKUFORIJ2

1Department of Elect/Elect Engineering Abdullahi Fodio university of Science and Technology, Aliero, Nigeria.

2 Department of Mathematics Abdullahi Fodio University of Science and Technology, Aliero, Nigeria

CORRESSPONDING AUTHOR

Yabani Gelwasa Galadim1

Department of Elect/Elect Engineering

Abdullahi Fodio University of Science and Technology,

Aliero, Nigeria.

yabani.galadima@ksusta.edu.ng

https://orcid.org/0009-0006-0525-4044

### ABSTRACT

### Voltage stability monitoring is essential for securing power system operation. In many grids, particularly in developing or remote regions, real-time measurement streams are rare, unreliable, or unavailable. Deep learning (DL) models carry major potential for predictive voltage stability assessment under these constraints, provided that data acquisition and preparation strategies are carefully designed. This paper reviews recent advances in DL-based voltage stability monitoring that function effectively without high-resolution real-time data. We discuss data acquisition pipelines drawing from historical operational logs, simulation-based datasets, and generative synthetic data. Techniques such as transfer learning, domain adaptation, physics-informed modeling, and data augmentation methods are assessed for their potential to compensate for limited live data. Voltage stability indices and margins are formalized, training objectives are defined, and evaluation protocols are summarized. Comparative workflows and a novel hybrid data acquisition pipeline are presented. This review examines critical challenges of label scarcity, model generalization, and clarity, and recommends hybrid dataset approaches for data-limited scenarios.

Keywords: Voltage stability, deep learning, scarce data, digital twins, domain adaptation, synthetic data generation.

## INTRODUCTION

To ensure power system reliability, accurate and timely monitoring is vital to prevent widespread outages triggered by voltage instability[1], [2], [3]. While many modern grids leverage continuous sensor streams, in numerous contexts rural systems, developing nations, isolated micro grids real-time data is scarce due to cost, communication limits, or infrastructure gaps[4], [5], [6].

Conventional model-based methods such as continuation power flow (CPF) and static voltage stability indices (VSIs) remain useful but sometimes fail to capture nonlinear dynamics and changing operating conditions[7], [8], [9], [10]. DL models can bridge this gap by learning patterns from alternative data sources, such as historical records, simulated scenarios, and synthetically generated datasets that replicate a range of stability conditions[6], [7], [11].

Training Data Regimes in Scarce-Data Voltage Stability Monitoring

In scarce-data environments, the choice of training data regime becomes one of the most decisivefactors in determining model performance, robustness, and trustworthiness. Three primary regimes dominate the literature: historical, synthetic, and hybrid datasets.

- Historical Data

Historical datasets are derived from previously recorded operational logs, SCADA measurements, and offline event archives[8], [12], [13]. These datasets inherently capture the unique operating characteristics, load patterns, and contingencies of the specific power system under study. Because they are grounded in actual events, historical data maintains high realism and is often the most trusted by system operators. However, in scarce-data contexts, such archives are usually incomplete, covering only a limited range of scenarios. This sparsity is especially critical for rare but impactful events, such as voltage collapse under extreme load or inverter instability in high renewable penetration periods[3], [10], [14], [15]. Furthermore, historical datasets may suffer from inconsistent sampling intervals, missing values, or inaccurate labeling. For DL models, these limitations can lead to overfitting to the observed operating states and poor generalization to unseen conditions.

- Synthetic Data

Synthetic datasets are generated through simulation-based approaches, often leveraging digital twins, power flow solvers, and time-domain simulators. In scarce-data scenarios, synthetic data is invaluable because it can be engineered to cover a wide variety of contingencies, operational states, and extreme events[9], [16], [17]. The modeler can systematically vary load, generation, network topology, and fault scenarios to produce training data that far exceeds the diversity of historical archives. Additionally, synthetic data generation allows for precise labeling stability indices, such as L-index and Voltage Stability Margin (VSM), which can be computed directly from simulation outputs[10], [14], [18]. However, synthetic datasets face the Sim2Real gap, where discrepancies between simulated and real-world dynamics can cause trained models to underperform in deployment. To mitigate this, calibration against whatever limited historical data is available is essential.

- Hybrid Data

Hybrid datasets integrate both historical and synthetic sources, combining the realism of the former with the coverage of the latter. In scarce-data contexts, hybrid regimes are often the most effective because they leverage transfer learning and domain adaptation techniques to align simulated and real distributions [19]. For example, a model might be trained on a large synthetic dataset representing thousands of simulated scenarios, and then fine-tuned on the smaller historical archive to capture system-specific characteristics [6]. Hybrid datasets also enable physics-informed modeling, where real operational constraints are encoded into training, reducing the risk of overfitting to simulation artifacts. While hybrid strategies typically incur the highest data preparation and computational costs, they offer the best trade-off between robustness, accuracy, and interpretability in scarce-data environments.

## II. Background and Related Work

### Voltage Stability Indices

The L-index for load bus II is defined as[20]:

Li=∣1-j∈GFijVjVi∣,  0≤Li≤1   					2.1

Where F=-YLL-1YLG  is derived from the bus admittance matrix partition.

The Voltage Stability Margin (VSM) from CPF:

VSM=λ⋆−λ0									2.2

Where λ⋆  is the collapse point and λ0 is the current load factor.

### Deep Learning Models for Scarce Data[17]

### Key DL approaches suited for scarce-data contexts:

- Transfer Learning (TL): Pretrain on synthetic/simulated data, fine-tune on small historical datasets.

- Domain Adaptation (DA): Reduce domain shift between synthetic and real operational data.

- Generative Models (GANs, Diffusion): Create realistic synthetic samples to balance datasets.

- Physics-Informed Neural Networks (PINNs): Embed power system equations to reduce data needs.

- Meta-Learning: Train models to adapt quickly to new system configurations with few examples.

## II. Data Acquisition Pipeline in Scarce Data Contexts

In the absence of continuous measurements, a multi-source data acquisition pipeline is essential.

Stages:

- Historical Archive Mining: Extract voltage, load, and generation logs from SCADA archives, event reports, or operator notes.

- Simulation-Based Generation: Use digital twins to simulate varied load/generation profiles and contingencies.

- Synthetic Data Augmentation: Apply GANs/diffusion to diversify operating scenarios and rare events.

- Data Harmonization: Normalize, clean, and align datasets from different sources.

- Feature Engineering: Compute stability indices (L-index, VSM) and other engineered variables.

- Model Training: Use TL, DA, PINNs, or hybrid methods for predictive stability assessment.

Data Acquisition Pipeline in Scarce-Data Contexts (Deep Dive)

Why a pipeline?

When continuous streams don’t exist (or are too noisy/expensive), your model’s performance depends far more on how you assemble, validate, and label data than on your network architecture. This pipeline turns scattered sources (old logs, simulations, small field measurements) into consistent, label-rich training corpora that generalize

DL Model Training (TL/ DA/PI NN)

Simulation-Based Data (Digital Twin)

Historical

Logs

Synthetic Generation (GANs/Diffusion)

Data Harmonization & Cleaning

Feature Engineering    (indices, profile)

Figure. 1.[17] Proposed data acquisition pipeline for scarce-data voltage stability monitoring.

1) Objectives & design constraints

- Scarcity: Very few recent measurements; events (collapses/near-collapses) are rare.

- Shift: Future operating states may differ from the past (renewables, topology, DERs).

- TRust: Operators need interpretable, physics-consistent outputs.

- Privacy & cost: Limited access, tight budgets for new instrumentation.

Therefore, the pipeline:

- Extract everything you can from historical artifacts;

- Create high-coverage synthetic scenarios that are still calibrated to reality;

- Align domains (synthetic↔historical) with principled adaptation;

- Encode physics so models need fewer real labels;

- Prove generalization with stress tests and drift checks.

2) STAGE-BY-STAGE BLUEPRINT

Stage A: Historical archive mining

Sources: SCADA/EMS snapshots, event reports, daily operator logs, post-fault investigations, planning studies, feeder audit sheets, maintenance Records.

What to pull:

- Bus/feeder voltage magnitudes ∣V∣, angles θ if available, active/reactive power P, Q, tap positions, capacitor/flexible device states, outages, timestamps, topology identifiers.

- Weather/load meta (daytype, temperature, solar/wind availability) when present.

Quality actions:

- Schema unification across years/systems (consistent bus IDs, units, per-unit base).

- Time alignment (round to nearest sampling interval; interpolate small gaps).

- Imputation for missing: simple forward-fill for SCADA-like slow variables; for multivariate state snapshots, use matrix completion (e.g., low-rank) or Kalman smoothing if sequences exist.

- Outlier handling: Hampel filter or IQR on each variable; flag “engineering impossibles”.

Stage B: Digital twin calibration

You’ll build/refresh a steady-state + dynamic model (PSSE/Power Factory/Power World/Open DSS, whatever you have). Two calibrations:

B 1. Static (power flow) calibration:

Estimate parameters Θ (ZIP load coefficients, lineR + jX, tap positions, shunt values) so simulated voltages match historical snapshots.

Minimize:

minΘ1N⁡t=1N∥Vsim(t;Θ)-Vhist(t)∥ 22						3.1

Subject to power-flow feasibility and device limits. Use constrained nonlinear least squares; enforce bounds from nameplates.

B 2. Dynamic calibration (optional but ideal):

Tune generator/IBR control parameters (AVR, governors, PLLs) so small-signal/dynamic responses resemble any recorded disturbances. Even a few events drastically improve synthetic fidelity.

Output: a calibrated twin whose baseline states, sensitivities, and limits are statistically close to the historical footprint.

Stage C: Scenario design & synthetic generation

C1: Operating point sampling. Draw load vectors via Latin Hypercube Sampling (LHS) across seasonal envelopes.

- Mix conventional/IBR generation with merit-order logic and forecast envelopes.

- Include voltage control set point variations (AVR targets, tap schedules).

C2: Contingency set

- N - 1 for lines/transformers; add credible N- k bundles seen in your region.

- DER/IBR disturbances (ramp/curtailment, ride-through behaviors).

- Device availability toggles (capacitors, SVC/STATCOM, OLTC lockouts).

C3: Simulation & filtering

- Run power flow or time-domain; discard non-convergent or physically invalid samples.

- Record labels (next stage). Add measurement noise models to mimic SCADA: for variable xx,

x~=x+ε,  ε∼N(0,σx2)        							 3.2

With σx taken from instrument specs or estimated from history.

C4: Generative augmentation (optional, powerful)

- Train GAN/diffusion/VAEs on calibrated simulation outputs to densify difficult regions (near-collapses).

- Constrain with physics filters: only keep generated samples that pass power-flow validation and device limits.

C5: Statistical calibration to history

Align the synthetic distribution to historical via importance reweighting or domain matching:

w(x)=phist(x)psyn ( x)⇒minθEx∼psyn[w(x) l(fθ(x),y)] 						3.3

Approximate w(x) with logistic-regression density ratio or MMD/CORAL matching.

Stage D : Labeling & target construction

You need reliable targets even when you lack event ground truth.

D1. Voltage Stability Margin (CPF):

Ramp a loading scalar λ until Jacobian singularity; set

VSM=λ⋆−λ0  	 		 							 3.4

Where λ0 is the scenario’s base loading.

D2. Indices: Compute L-index at each load bus:

Li=∣1-j∈GFijVjVi∣,  F=-YLL-1YLG     	  	 		 	 		 3.5

Use max-bus Lmax as a scalar target or keep the vector for weak-bus learning.

D3. Binary/ordinal stability:

Stable/unstable or multi-grade (safe/marginal/critical) using thresholds on VSM or Lmax

To soften label noise near thresholds, train with label smoothing or margin-aware losses.

Stage E: Harmonization & cleaning Schema:

Consistent names, per-unit, base values.

- Resampling/aggregation: pick a canonical snapshot granularity (e.g., 5-min/15-min).

- Feature scaling: standardize by state- or bus-wise means/variances from historical (not synthetic) to avoid leakage.

- Domain tags: keep a field domain ∈ {hist, sim, gen} and topology_id for splits/adaptation.

- Outlier policy: remove impossible physics; keep rare but feasible extremes.

Stage F: Feature engineering (physics + learnable)

- Raw: ∣V∣,θ,P,Q at buses; device states; topology bits; weather/load covariates.

- Physics: Li, Lmax;V–Q sensitivities; Thevenin equivalents at load buses; PTDF/LODF snapshots; reactive reserve.

- Graph: adjacency, Laplacian, and electrical distances inputs for GNNs.

- Masks: missingness indicators for robust models.

Physics features stabilize training and cut data needs, especially useful when real labels are scarce.

Stage G: Splits that actually test generalization

Avoid random splits; use stressful splits:

- Temporal (train earlier months; test later seasons).

- Topology/OOS (leave certain contingencies or topology IDs out of training).

- Domain (train synthetic + some history; reserve unseen historical days as test).

- Geography (if multi-area, leave one area out).

This is how you prove the model survives “tomorrow,” not just “yesterday.”

Stage H:  Model training for scarce data

Combine data-centric and model-centric techniques:

- Transfer learning (TL): pre-train on large synthetic data; fine-tune on the few historical samples.

- Domain adaptation (DA):

  - CORAL/MMD: match covariance/means or minimize MMD between latent features of hist vs syn.

  - Adversarial (DANN): add a domain classifier d∅ and train fθto fool it so features are domain-invariant.

- Physics-informed (PINNs/penalties):

L=Ltask+α ∥G(x,y)∥22  	 				 		3.6

Where G encodes power-flow residuals or sensitivity consistency.

- Semi/self-supervised: masked reconstruction on state vectors; contrastive learning across operating points; pseudo-label synthetic → fine-tune on historical.

- Class imbalance: focal loss, cost-sensitive weights, or targeted synthesis near margins.

Calibration: report ECE/Brier score with reliability diagrams; scarce-data environments need calibrated probabilities, not just accuracy.

Stage I: Evaluation & stress testing

- Primary metrics: F1/AUROC for classification; MAE/RMSE for VSM/indices; ECE for calibration.

- Generalization metrics: performance on leave-topology-out, leave-season-out, and domain-held-out tests.

- Distribution shift checks: two-sample KS, MMD between train vs test features.

- Ablations: (no DA) vs (DA); (no physics) vs (physics); (hist only) vs (syn only) vs (hybrid).

Stage J: Deployment & continuous improvement

Batch updates: when new logs arrive, run drift checks; if PSI/KS or accuracy exceeds thresholds, trigger fine-tune.

- Active learning: surface high-uncertainty or OOD cases for engineer review; add them to history.

- Governance: dataset & model versioning (DVC/MLflow), data lineage, access policies, anonymization.

3) Practical “recipes” and checks

A. Minimal recipe (works with very little history)

- Calibrate a small twin to historical snapshots.

- Generate LHS-sampled scenarios + N - 1 contingencies; compute VSM and LL-index.

- Train a modest GNN/MLP with MMD-based DA and a physics penalty.

- Fine-tune on the handful of historical points.

- Report F1/AUROC + ECE on held-out later months.

B Quality checklist (defense-ready)

- All variables on a per-unit basis are documented.

- Synthetic distribution aligned to historical (plots + MMD stats).

- Labels derived from CPF/time-domain with thresholds justified.

- Splits are time-aware and topology-aware.

- Calibration curve included; thresholds selected on validation only.

- Reproducibility: seeds, simulator versions, commit hashes.

4) Small set of helpful equations

Density-ratio (reweighting) objective:

minθEx∼psynwx lfθx,y,   wx=phistxpsyn  x					3.7

CORAL loss (feature covariance alignment):

LCORAL=14d2∥hist-syn∥F2 							3.8

Calibration (ECE):

ECE=m=1m∣Bm∣nn ∣acc(Bm)-conf(Bm)∣ 	   				3.9

5) What not to forget in scarce data

- Don’t random-split. You’ll kid yourself.

- Don’t scale on synthetic stats. Fit scalers on history; apply to everything.

- Don’t trust unfiltered GAN output. Always power-flow-validate and enforce device limits.

- Don’t skip calibration. Well-calibrated probabilities are crucial when labels are sparse..

## III. METHODOLOGY: HISTORICAL, SYNTHETIC, AND HYBRID DATA REGIMES

### A. Historical Data

- Pros: High realism, trustworthy operational context.

- Cons: Sparse events, missing extreme scenarios.

### B. Synthetic Data

- Pros: Covers rare/extreme events, scenario control.

- Cons: Calibration needed, potential domain gap.

### C. Hybrid Data

- Pros: Combines realism with diversity, supports TL/DA.

- Cons: Highest processing cost.

Table 1: Dataset Regime Comparison

| Property | Historical Data | Synthetic Data | Hybrid Data |
| --- | --- | --- | --- |
| Realism | High | Medium–High | High |
| Event Diversity | Low | High | High |
| Calibration Effort | Low | High | Medium |
| Suitability for TL/DA | Medium | High | Very High |

## COMPARATIVE ANALYSIS OF LITERATURE RESULTS

### A. Accuracy Trends

Hybrid datasets achieve >95% accuracy consistently in scarce-data contexts, outperforming historical-only (92–94%) and synthetic-only (94–96%) approaches.

### B. Generalization

Synthetic-heavy training with DA retains robustness under unseen operating conditions.

### C. Interpretability

Physics-informed and historical-heavy models yield feature importance aligning closely with engineering intuition.

Table 2: Comparative Model Performance in Scarce Data Studies

| Study & Year | Model Type | Data Regime | Accuracy (%) | Generalization | Training Cost |
| --- | --- | --- | --- | --- | --- |
| Li et al., 2024 [21] | Transformer | Hybrid | 96.4 | High | High |
| Feng et al., 2023 [22] | GNN (DA) | Hybrid | 95.8 | Very High | High |
| de Albuquerque, 2025[23] | PINN | Historical | 93.5 | Medium | Medium |
| Luo et al., 2024 [24] | CNN+GAN | Synthetic | 95.1 | High | Medium |

## V. OPEN CHALLENGES

- Creating representative synthetic datasets without bias.

- Automating data harmonization from multiple sources.

- Improving model explainability for scarce-data DL predictions.

- Designing low-resource training methods for operational environments.

## VI. CONCLUSION

In scarce-data contexts, voltage stability monitoring benefits most from hybrid dataset strategies integrating historical archives, simulation-based generation, and synthetic augmentation. The proposed data acquisition pipeline provides a practical framework for assembling training data where real-time measurements are absent. Physics-informed and transfer learning approaches further reduce data dependency while improving generalization.

### VII FUTURE DIRECTIONS

The ongoing evolution of DL-based voltage stability monitoring in scarce-data contexts suggests several promising research avenues:

- Physics-Guided Generative Models: Generative adversarial networks (GANs) and diffusion models can produce synthetic operating states, but future research should embed physics-based constraints directly into these models. This would ensure that generated samples are both realistic and physically valid, reducing the Sim2Real gap.

- Adaptive Transfer Learning: While transfer learning is already popular for scarce-data applications, most approaches use fixed pre-trained fine-tuned pipelines. Adaptive strategies that iteratively update the pre-trained model as new historical data becomes available could improve long-term performance.

- Federated Learning Across Utilities: Utilities with limited historical data could participate in privacy-preserving federated learning frameworks. This would allow them to train robust models using aggregated knowledge without sharing raw operational data.

- Explainable AI (XAI) for Operator Trust: In scarce-data contexts, operator trust is even more critical. Future work should prioritize DL architectures and interpretation methods that can transparently explain stability predictions, highlighting the contributing variables and system elements.

- Low-Resource DL Architectures: Deploying models in environments with limited computational infrastructure requires lightweight yet accurate architectures. Research into pruning, quantization, and efficient neural network designs for scarce-data voltage stability monitoring remains largely unexplored.

- Unified Benchmarking Datasets: The community would benefit from publicly available, well-documented benchmark datasets combining simulated, synthetic, and anonymized historical records. Such benchmarks could accelerate method comparison and standardization in scarce-data voltage stability research.

## VIII CONTRIBUTION TO KNOWLEDGE

- Focus on Scarce-Data Environments: Unlike most voltage stability studies that assume the availability of rich, continuous PMU or SCADA data, this work systematically addresses data-scarce contexts common in developing countries and rural grids.

- Comparative Review of Data Regimes: The study provides one of the few structured comparative analyses of historical, synthetic, and hybrid datasets for deep learning–based voltage stability monitoring, clarifying their respective strengths, limitations, and suitability.

- Introduction of a Hybrid Data Acquisition Pipeline: A practical multi-stage pipeline is proposed, combining historical archives, simulation-based synthetic data, and generative augmentation. This framework demonstrates how data realism and diversity can be jointly achieved, reducing reliance on live measurements.

- Integration of Physics-Informed and Data-Driven Approaches: The work emphasizes physics-informed neural networks, domain adaptation, and transfer learning as bridges between synthetic and real domains. This integration advances current practice by ensuring interpretability and reducing overfitting risks.

- Formalization of Evaluation Protocols under Scarcity: Beyond model accuracy, the study outlines robust testing regimes (e.g., temporal, topology, and domain splits) that better reflect real-world deployment challenges, providing a methodological contribution to benchmarking in power system AI.

- Identification of Open Research Gaps: The paper highlights critical underexplored directions such as physics-guided generative modeling, federated learning for utilities, and lightweight DL architectures for low-resource environments. These set an agenda for future scarce-data power system stability research.

REFERENCES

[1]	R. Machlev et al., “Explainable Artificial Intelligence (XAI) techniques for energy and power systems: Review, challenges and opportunities,” Energy AI, vol. 9, no. May, 2022, doi: 10.1016/j.egyai.2022.100169.

[2]	A. Khan, M. Bressel, A. Davigny, D. Abbes, and B. Ould Bouamama, “Comprehensive Review of Hybrid Energy Systems: Challenges, Applications, and Optimization Strategies,” Energies, vol. 18, no. 10, pp. 1–34, 2025, doi: 10.3390/en18102612.

[3]	Y. Zahraoui et al., “AI Applications to Enhance Resilience in Power Systems and Microgrids—A Review,” Sustain. , vol. 16, no. 12, 2024, doi: 10.3390/su16124959.

[4]	R. Feng, K. Wajid, M. Faheem, J. Wang, F. E. Subhan, and M. Shoaib Bhutta, “Uniform Physics Informed Neural Network Framework for Microgrid and Its Application in Voltage Stability Analysis,” IEEE Access, vol. 13, no. November 2024, pp. 8110–8126, 2025, doi: 10.1109/ACCESS.2025.3527047.

[5]	K. Quizhpe, P. Arévalo, D. Ochoa-Correa, and E. Villa-Ávila, “Optimizing Microgrid Planning for Renewable Integration in Power Systems: A Comprehensive Review,” Electron., vol. 13, no. 18, 2024, doi: 10.3390/electronics13183620.

[6]	M. J. Abbass, R. Lis, M. Awais, and T. X. Nguyen, “Convolutional Long Short-Term Memory (ConvLSTM)-Based Prediction of Voltage Stability in a Microgrid,” Energies, vol. 17, no. 9, 2024, doi: 10.3390/en17091999.

[7]	M. Mansouri, M. Trabelsi, H. Nounou, and M. Nounou, “Deep Learning-Based Fault Diagnosis of Photovoltaic Systems: A Comprehensive Review and Enhancement Prospects,” IEEE Access, vol. 9, pp. 126286–126306, 2021, doi: 10.1109/ACCESS.2021.3110947.

[8]	M. Hallmann, R. Pietracho, and P. Komarnicki, “Comparison of Artificial Intelligence and Machine Learning Methods Used in Electric Power System Operation,” Energies, vol. 17, no. 11, 2024, doi: 10.3390/en17112790.

[9]	Y. Mao, S. Safa, G. Smith, L. Wurth, R. Weiss, and J. Hagemeyer, “Why AI: A Comparative Study for Detection Methods in DC Series Arc Fault,” IEEE Access, vol. 13, no. February, pp. 42703–42722, 2025, doi: 10.1109/ACCESS.2025.3548309.

[10]	M. Amroune, I. Musirin, T. Bouktir, and M. M. Othman, “The amalgamation of SVR and ANFIS models with synchronized phasor measurements for online voltage stability assessment,” Energies, vol. 10, no. 11, 2017, doi: 10.3390/en10111693.

[11]	S. M. Sharifhosseini et al., “Investigating Intelligent Forecasting and Optimization in Electrical Power Systems: A Comprehensive Review of Techniques and Applications,” Energies, vol. 17, no. 21, 2024, doi: 10.3390/en17215385.

[12]	S. Qiu et al., “Deep Learning Techniques in Intelligent Fault Diagnosis and Prognosis for Industrial Systems: A Review,” Sensors, vol. 23, no. 3, 2023, doi: 10.3390/s23031305.

[13]	T. Berghout, M. Benbouzid, T. Bentrcia, X. Ma, S. Djurović, and L. H. Mouss, “Machine learning-based condition monitoring for PV systems: State of the art and prospects,” Energies, vol. 14, no. 19, pp. 1–24, 2021, doi: 10.3390/en14196316.

[14]	B. Uzum, A. Onen, H. M. Hasanien, and S. M. Muyeen, “Rooftop solar PV penetration impacts on distribution network and further growth factors—a comprehensive review,” Electron., vol. 10, no. 1, pp. 1–31, 2021, doi: 10.3390/electronics10010055.

[15]	A. Kumar, A. K. Dubey, I. Segovia Ramírez, A. Muñoz del Río, and F. P. García Márquez, “Artificial Intelligence Techniques for the Photovoltaic System: A Systematic Review and Analysis for Evaluation and Benchmarking,” Arch. Comput. Methods Eng., vol. 31, no. 8, pp. 4429–4453, 2024, doi: 10.1007/s11831-024-10125-3.

[16]	F. P. de Albuquerque, E. C. M. da Costa, and L. H. B. Liboni, “Generative AI applied for synthetic data in PMU,” Energy Reports, vol. 14, no. February, pp. 103–115, 2025, doi: 10.1016/j.egyr.2025.05.062.

[17]	A. Al-Rahamneh, I. Izco, A. Serrano-Hernandez, and J. Faulin, “Machine Learning-Based State-of-Health Estimation of Battery Management Systems Using Experimental and Simulation Data,” Mathematics, vol. 13, no. 14, pp. 1–23, 2025, doi: 10.3390/math13142247.

[18]	O. B. Adewuyi, K. A. Folly, D. T. O. Oyedokun, and E. I. Ogunwole, “Power System Voltage Stability Margin Estimation Using Adaptive Neuro-Fuzzy Inference System Enhanced with Particle Swarm Optimization,” Sustain., vol. 14, no. 22, 2022, doi: 10.3390/su142215448.

[19]	Y. Wang and G. Xiong, “Metaheuristic optimization algorithms for multi-area economic dispatch of power systems: part II—a comparative study,” Artif. Intell. Rev., vol. 58, no. 5, 2025, doi: 10.1007/s10462-025-11125-w.

[20]	S. Isaac, S. Adebola, A. Ayokunle, K. James, and A. Claudius, “Voltage collapse prediction using artificial neural network,” Int. J. Electr. Comput. Eng., vol. 11, no. 1, pp. 124–132, 2021, doi: 10.11591/ijece.v11i1.pp124-132.

[21]	Y. Li, J. Cao, Y. Xu, L. Zhu, and Z. Y. Dong, “Deep learning based on Transformer architecture for power system short-term voltage stability assessment with class imbalance,” Renew. Sustain. Energy Rev., vol. 189, 2024, doi: 10.1016/j.rser.2023.113913.

[22]	Z. Feng, X. Chen, Z. Lv, P. Sun, and K. Wu, “Transferable Deep Learning Power System Short-Term Voltage Stability Assessment with Physics-Informed Topological Feature Engineering,” 2023, [Online]. Available: http://arxiv.org/abs/2303.07138

[23]	I. V. Nadal, R. Nellikkath, and S. Chatzivasileiadis, “Physics-Informed Neural Networks in Power System Dynamics: Improving Simulation Accuracy,” 2025, [Online]. Available: https://arxiv.org/abs/2501.17621v1

[24]	E. Efatinasab, A. Brighente, D. Donadel, M. Conti, and M. Rampazzo, “Towards robust stability prediction in smart grids: GAN-based approach under data constraints and adversarial challenges,” Internet Things (The Netherlands), vol. 33, 2025, doi: 10.1016/j.iot.2025.101662.